motion detecting email screenshots operation

1 set photo storage

1.1 click on the path

The main menu - > video recording - > image storage

1.2 set the photo storage

1. The channel can choose, if you want to choose all, channel selection,

2. Take the default,

3. Capture mode selection configuration,

Select all 4 weeks, time can be set up according to the requirements

5. Capture type selection test, ordinary and alarm can be canceled, and then point to determine,

2 set the motion detecting

2.1 click on the path

The main menu - > - > mobile detection alarm function

2.2 set up the linkage

1. The regional and time can choose to trigger the alarm time

2. Video channel and grasp the figure indicates the channel trigger the alarm, will trigger the selected channel videos and screenshots,

3. Check the Email, you can send alarm message by Email,

Note: alarm mail with pictures, only can't attached video

3 set up

3.1 click on the path

Main menu - > Settings - > network services - > Email Settings

3.2 set up the mailbox

1) the SMTP server, set to smtp.xxx.com format, QQ mailbox set to smtp.exmail.xxx.com format, XXX on behalf of the mailbox type

2) foreign customers to check the SSL

3) user name same to the sender, best can be written as a mailbox full name

4) the recipient can be the same as the sender, you can set up multiple recipients, in English a semicolon

5) subject to modification, set equipment name or location is convenient to distinguish

6) information to fill out completely can click mail test, verify the Settings are correct and can be connected

Note: the email address set up the network need to make sure that equipment can be connected in the network; Support email main currently has around 126, 163, gmail, sina, 139, 156, QQ format;

The recipient can set up to five email effectively;