1  the IPC camera default IP address

The camera to factory default IP for: 192.168.1.10

Note: access device for the first time, please modify IP and local PC to a network segment

2  PCS in the same local area network (LAN) in the search is less than the IP address of the camera

a - check camera tail line cable interface, green light, the device has access to electricity, no light, please replace the power supply or socket; Yellow light, network connectivity, yellow light is not bright, replace the cable or Ethernet cable socket

b - with a network cable directly connected with computer, the computer is set to a fixed IP address, only a local connection, wireless and other banned them first

c -- if you can search, please check the router and switch network configuration

d - if no, in the case of known device IP address, ping the IP equipment

e - if you won't be able to search, contact the seller technical processing

3  the IPC how to adjust image color

a - the browser login, can be seen in the other set in the lower right corner image color options, click open interface to set the related parameters,

b -- video access, right-clicking on channel images, select the image color, modify the parameters

c - now our back-end VCR to IPC, image color, can be set directly on the back end can also be synchronized to the front on the IPC

Note: the parameters including brightness, contrast, saturation, hue, gain, horizontal sharpness, vertical sharpness

4  the IPC camera can search to two IP addresses

- if the device to connect the WIFI module, search will appear two IP, is a cable, a is the WIFI

5  the IPC upgrade process network suddenly or without electricity

a-100% if the data has been sent, upgrade process power failure; Can lead to equipment cannot be started, need to a serial port upgrade or burn flash or returned to the factory

b-100% if the data has been sent, the upgrade process interrupt network, is not affected, but can't see the hint of promotion, there is a certain risk, also may not be able to start)

c -- if the data is not sent by 100%, the way power or broken network, will not affect

6  the IPC through IE can only display a picture, how many IPC preview at the same time

- through the web access the IPC, can only display a picture

- there are many sets of IPC, with supporting our client software (such as: CMS VMS SNVR)